Using Xilinx Foundation Series Software:

Observation of Functional Simulation within the Schematic Editor

- To observe the simulation in the Schematic Editor window, the circuit has to have been set up for Waveform Viewer simulation, this includes adding signals, stimulus, and clock frequency steps in the Sim Funct window.  See: Using Xilinx Foundation Series Software:  Functional Simulation in the Waveform Viewer

- In the Win95 taskbar or the Xilinx Program Manager, open the Schematic Editor.  The Schematic Editor window should appear with colored boxes (probes) representing the value (0 or1) at that point in the circuit.

- To control the simulation from the Schematic Editor window you will need the Simulation Toolbox, its button is on the top toolbar, and looks like red and green lights.  Click on it and the SC Probes toolbox will appear.

- To step the simulation forward select the Next Event button in the SC Probes toolbox.  The values in the colored boxes should change to their next values.

- It is useful to add probes to the circuit to understand the way the signals propagate through the gates and buffers.  To add probes, select the Probe Tool in the SC Probes toolbox, and place them in the circuit by clicking on various  points in the circuit.

- Step the simulation forward a couple of times to observe the propagation of the values through the logic circuit.  Note:  This can be extremely helpful when debugging a circuit that doesn’t work correctly.  The probes make it obvious where the logic is wrong, or the wires are not correctly connected.

- At any time you can switch to the Waveform Viewer window by selecting the SIM button, it is next to the Simulation Toolbox button on the top toolbar.  The SIM button is also in the SC Probes toolbox.